1.0 Networking Concepts

| OSI Layer | Layer Name | Data Unit | Common Protocols & Technologies |
| --- | --- | --- | --- |
| 7 | Application | Data | HTTP, HTTPS, FTP, SMTP, IMAP, POP3, DNS, SNMP |
| 6 | Presentation | Data | TLS/SSL, JPEG, MPEG, GIF, ASCII, EBCDIC |
| 5 | Session | Data | NetBIOS, PPTP, RPC, SMB session control |
| 4 | Transport | Segment | TCP, UDP, SCTP |
| 3 | Network | Packet | IP, ICMP, IGMP, IPSec, OSPF, RIP, BGP |
| 2 | Data Link | Frame | Ethernet, PPP, ARP, MAC, VLAN (802.1Q), STP |
| 1 | Physical | Bits | Cables, Hubs, Wi-Fi (802.11), Bluetooth, DSL, Fiber, modulation (e.g., QAM, OFDM) |

| Feature | SAN | NAS |
| --- | --- | --- |
| Storage Level | Block-level | File-level |
| Network Type | Dedicated Storage Network (Fiber Channel, iSCSI) | Standard LAN (Ethernet) |
| Access Method | Acts like a local hard drive to servers | Accessed as shared folders/files |
| Performance | High throughput, low latency | Lower throughput, higher latency |
| Complexity | More complex, requires specialized hardware | Easier to set up and manage |
| Typical Use | Enterprise data centers, databases | File sharing, backups, media storage |

Content Delivery Network –

A content delivery network is a globally distributed network of servers that delivers web content to users based on their geographic location. It caches content on servers called edge servers and delivers content from the closest edge serve to the user.

TTL (Time to Live) –

Time to live is a field in the IP packet header that limits the lifespan of a packet on a network. This prevents packets from circling endlessly in routing loops and discards them if they can’t reach their destination. IP packets have TTL values that typically start at 64, 128, or 255. Every router that handles the packet decreases the TTL value by one. If the packet reaches a TTL value of 0, it is dropped, and an ICMP “Time Exceeded” message is sent back to the sender.

NFV (Network Functions Virtualization) –

NFV is a way to virtualize entire classes of network functions like firewalls, routers, and load balancers. Instead of hardware, they run on software on standard servers. Common examples are IDS/IPS, virtual routers, and virtual firewalls. NFV is easy to scale and deploy new functions.

VPC (Virtual Private Cloud) –

Isolated from other customers, your own personal cloud. It is like having your own data center within a cloud provider. Customizable with security, routing and segmentation.

NSG (Network Security Group) –

NSG’s are firewall-like security rules that control inbound and outbound traffic to resources inside of a VPC. Rules are based on source IP, destination IP, Port, and Protocol. Allow or deny traffic to virtual machines.

NSL (Network Security Lists –

These are like NSG’s, but they have stateless rules. If you allow inbound traffic, you must also explicitly allow outbound return traffic. The stateless firewall rules are at a subnet level.

SaaS (Software as a Service) –

Software as a Service provides cloud hosted applications for users to log in and use. The cloud provider handles everything else like the OS, hardware, and software. Examples include Gmail, ms365, Dropbox, and Zoom.

PaaS (Platform as a Service) –

Platform as a Service allows you to build and deploy your apps while the provider handles the underlying infrastructure. Provides the environment to write and deploy code.

IaaS (Infrastructure as a Service) –

With Infrastructure as a Service, you rent out raw infrastructure such as VMs, storage, and networks while configuring it yourself. The provider handles the physical hardware and the virtualization of said hardware.

| Feature | Scalability | Elasticity | Multitenancy |
| --- | --- | --- | --- |
| 🔍 Definition | Ability to increase resources to meet growing demand | Ability to automatically add/remove resources based on usage | Multiple customers (tenants) share the same physical/cloud resources |
| ⚙️ How it works | Add more capacity manually or automatically | Resources adjust in real-time based on demand | Isolation is logical (not physical); each user sees only their data/services |
| ↕️ Vertical Scaling | Add CPU/RAM to existing systems | Not typically used | Not applicable |
| ↔️ Horizontal Scaling | Add more systems (e.g., web servers) | Yes, often done automatically | Not applicable |
| 🤖 Automation | Optional (can be manual or auto) | Yes — usually triggered by load metrics | Automatic separation by the cloud platform |
| 💡 Example | Adding 5 new web servers when users grow | Auto-scaling VMs up/down during traffic spikes | Microsoft 365 serving multiple companies from the same cloud infrastructure |
| 📝 Exam Keywords | “Increase capacity”, “Handle more users” | “Dynamically adjusts”, “Real-time scaling” | “Shared infrastructure”, “Tenant isolation”, “Cloud resource sharing” |

File Transfer Protocol (FTP) 						20/21

Secure File Transfer Protocol (SFTP) 				22

Secure Shell (SSH)							22

Telnet 									23

Simple Mail Transfer Protocol (SMTP) 				25

Domain Name System (DNS) 					53

Dynamic Host Configuration Protocol (DHCP) 			67/68

Trivial File Transfer Protocol (TFTP) 					69

Hypertext Transfer Protocol (HTTP) 					80

Network Time Protocol (NTP) 					123

Simple Network Management Protocol (SNMP) 			161/162

Lightweight Directory Access Protocol (LDAP) 			389

Hypertext Transfer Protocol Secure (HTTPS) 			443

Server Message Block (SMB) 					445

Syslog 									514

Simple Mail Transfer Protocol Secure (SMTPS) 			587

Lightweight Directory Access Protocol over SSL (LDAPS) 	636

Structured Query Language (SQL) Server 				1433

Remote Desktop Protocol (RDP) 					3389

Session Initiation Protocol (SIP) 					5060/5061

GRE (Generic Routing Encapsulation) –

GRE allows for the transport of packets that are not natively supported by the underlying network. It is a layer 3 tunneling protocol. It creates point-to-point tunnels over IP networks. Connect remote sites over public networks and enables routing protocols and multicast traffic.

IPsec (Internet Protocol Security) –

IPsec is a protocol suite that provides secure communication over IP networks by authenticating and encrypting each IP packet of a data stream. The key functions of IPsec are authentication, encryption, integrity, and anti-replay. IPsec works at Network Layer 3 of the OSI model. It can create VPN tunnels over public networks and is commonly paired with protocols like GRE to tunnel non-IP or multicast traffic securely. IPsec protocols include:

Authentication Header – Provides authentication and integrity but no encryption.

Encapsulating Security Payload – Provides encryption, authentication, and integrity.

Internet Key Exchange – Protocol to establish and manage security associations.

| Traffic Type | Destination | How It Works | Use Case Example | Key Notes |
| --- | --- | --- | --- | --- |
| Unicast | One specific device | One-to-one communication between a single sender and receiver | Browsing a website, sending an email | Most common traffic type |
| Multicast | Group of devices | One-to-many communication, only to interested hosts (in a group) | Streaming video to multiple subscribers | Uses Class D IPs (224.0.0.0 – 239.255.255.255) |
| Anycast | Nearest of many devices | One-to-nearest (in terms of routing distance) of multiple receivers | DNS request to the closest server | Often used in IPv6 & CDNs |
| Broadcast | All devices on the network | One-to-all communication in the broadcast domain | ARP requests, DHCP discovery | Works only in IPv4, not in IPv6 |

| Standard | Frequency | Max Speed | Max Range (Indoor) | Channel Width | Notes |
| --- | --- | --- | --- | --- | --- |
| 802.11a | 5 GHz | 54 Mbps | ~35 m (115 ft) | 20 MHz | Less interference, shorter range |
| 802.11b | 2.4 GHz | 11 Mbps | ~35 m (115 ft) | 22 MHz | Longer range, but slower and more interference |
| 802.11g | 2.4 GHz | 54 Mbps | ~38 m (125 ft) | 20 MHz | Backward compatible with 802.11b |
| 802.11n | 2.4 & 5 GHz (dual) | 600 Mbps (theoretical) | ~70 m (230 ft) | 20/40 MHz | Uses MIMO (up to 4 streams) |
| 802.11ac | 5 GHz | 1.3 Gbps+ | ~35 m (115 ft) | 20/40/80/160 MHz | Uses MU-MIMO, beamforming |
| 802.11ax (Wi-Fi 6) | 2.4 & 5 GHz | 9.6 Gbps (theoretical) | ~35–70 m | 20–160 MHz | Better performance in crowded environments |
| 802.11be (Wi-Fi 7)* | 2.4 / 5 / 6 GHz | Up to 46 Gbps* | TBD | 320 MHz* | *Not on Net+ yet — emerging tech |

Satellite –

Wireless WAN up to 100 Mbps. It is used in rural areas and is affected by weather and has high latency.

Microwave –

Point-to-point up to 1 Gbps+. Long distance telecom and requires line of site.

WiMAX –

Fixed wireless up to 1 gig. Largely phased out but still relevant. Wireless ISP (WISP), city-wide broadband.

| Standard | Name | Speed | Medium | Max Distance | Common Use |
| --- | --- | --- | --- | --- | --- |
| 802.3 | Ethernet | 10 Mbps | Coaxial (thicknet/thinnet) | 500 m (thicknet) / 185 m (thinnet) | Legacy Ethernet (obsolete) |
| 802.3u | Fast Ethernet | 100 Mbps | UTP (Cat5+) or Fiber | 100 m (UTP) / 2 km (fiber) | Office LANs |
| 802.3z | Gigabit Ethernet (Fiber) | 1 Gbps | Fiber optic | Up to 5 km (single-mode) | Data centers, backbone links |
| 802.3ab | Gigabit Ethernet (Copper) | 1 Gbps | UTP (Cat5e+) | 100 m | Modern office & enterprise LANs |
| 802.3ae | 10 Gigabit Ethernet | 10 Gbps | Fiber optic | 26 m (MMF) to 40 km (SMF) | High-performance networks |
| 802.3an | 10GBase-T | 10 Gbps | UTP (Cat6a+) | 100 m | Cheaper 10G in copper environments |
| 802.3bz | 2.5G / 5G Ethernet | 2.5 / 5 Gbps | UTP (Cat5e/Cat6) | 100 m | Upgrade over Cat5e without re-cabling |
| 802.3by | 25G Ethernet | 25 Gbps | Fiber / Twinax | Short distances (DC use) | High-speed server uplinks |
| 802.3cn | 50G / 100G Ethernet | 50 / 100 Gbps | Fiber | Data center focused | Hyperscale, cloud backbone |
| 802.3cg | 10 Mbps (Long Reach) | 10 Mbps | Single pair copper | 1 km+ | IoT, industrial automation (10Base-T1L) |

SDN/SD-WAN (Software Defined Networking) –

The key concepts to SDN/SD-WAN is that it has application aware routing, zero-touch provisioning, and central policy management. IT simplifies management of large networks, enables efficient routing for cloud apps, and reduces human error with automation.

VXLAN –

Encapsulates Layer 2 over Layer 3. VXLAN scales virtual networks across data centers. Enables cloud-based VM migrations. Think about a virtual VLAN that stretches across the internet. It encapsulates ethernet frames inside UDP packets so that virtual machines in different data centers or network segments can communicate as if they were on the same LAN. Uses involve DCI (data center interconnect), large scale cloud environments, and hybrid cloud.

VTP (VLAN Trunking Protocol) –

VLAN Trunking Protocol allows a VLAN created on one switch to be propagated to other switches in a group of switches in a VTP domain.

ZTA (Zero Trust Architecture) –

Never trust, always verify. In a zero-trust architecture, it is set to deny by default. It enhances security for remote and internal users.

IaC (Infrastructure as Code) –

The practice of managing infrastructure using code and automation tools instead of manual configurations. Playbooks/Templates are pre-written scripts or config files to deploy setups. Config drift is when a device’s actual config drifts from the intended state. Faster deployment and eliminating human error, zero touch provisioning.

SASE/SSE (Secure Access Security Edge) (Security Service Edge) –

Secure Access Security edge is a cloud-delivered security + networking model. It merges SD-WAN with a full suite of security tools delivered from the cloud.

Security Service Edge is the security-only portion of SASE (SASE minus the networking).

2.0 Network Implementation

Subinterfaces –

A subinterface is a logical (virtual) interface created on a single physical interface of a router. It allows one physical port to be used for multiple virtual connections. Each subinterface will be assigned its own IP address, VLAN ID, and config. This is primarily used in Inter-VLAN routing.

Mesh Network –

Every node connects to every other node. Mesh networks are very expensive but offer better redundancy.

Ad-Hoc Network –

Peer-to-peer wireless communication. There is no central control in this type of network. This type of network is typically temporary, like devices connecting to each other directly for file sharing.

Point-to-point Network –

In a P2P network, one device is directly connected to another. This is often used in WAN links like leased lines.

Infrastructure Mode –

Infrastructure mode uses a central AP to control traffic. This type of network is the most common type when it comes to Wi-Fi deployment in homes/offices.

Autonomous AP’s –

These are standalone units where each one is configured individually. They include their own management, DHCP, and sometimes even routing.

Lightweight AP’s –

These AP’s are managed by a Wireless LAN Controller (WLC). They offer centralized management and are typically used in enterprise environments.

Rack size –

Rack size is measured in rack units (U) – 1U = 1.75 inches. Common rack sizes are 42U, 48U. The standard rack width is 19 inches.

Port-Side Exhaust/Intake –

Many devices vent air front to back. Important for airflow alignment in data centers. All devices should follow the same intake/exhaust pattern to prevent hot air recirculation.

Patch Panel –

A patch panel is an organized way to connect user ports to switch ports. It provides flexibility and easy reconfiguration. They streamline cable management, reduce clutter, and provide a structured approach to cable organization within the rack.

Clean Agent –

This type of fire suppression uses chemical agents that leave no residue. It is electrically nonconductive and safe for electronics. This is ideal for IT environments.

Inert Gas Systems –

Inert Gas Systems use natural inert gases to suppress fire by lowering oxygen levels. There is no chemical reaction, instead, they suffocate the fire. While it is safe for electronics, it is not safe for people in high concentrations.

3.0 Network Operations

Change Management –

The purpose of the change management process is to ensure that changes are approved, tested, and documented. Important factors include rollback/recovery plans if something went wrong. This helps prevent outages from poorly planned changes.

| Field | Description |
| --- | --- |
| Change description | What exactly is being changed |
| Reason/justification | Why the change is necessary |
| Start/end time | When the change will be performed |
| Affected systems | Devices, services, or users impacted |
| Rollback plan | Steps to revert if something fails |
| Approval chain | Names/titles of approvers |

| Step | What Happens |
| --- | --- |
| 📝 Request submission | User or technician creates a service ticket |
| 📋 Initial review | Ticket assigned, classified (priority, type) |
| 🔁 Action taken | Change implemented, tested, validated |
| ✅ Closure | Ticket marked resolved and closed |
| 🗃️ Auditable history | Full record for tracking and compliance |

SNMP (Simple Network Management Protocol) –

SNMP is used to monitor and manage network devices. It allows a central system (SNMP Manager) to gather performance, data, and status info from networked devices (SNMP Agents). SNMP operates at the application layer of the OSI model.

| Term | Description |
| --- | --- |
| SNMP Manager | The system that polls agents and receives trap alerts (e.g., SolarWinds, PRTG) |
| SNMP Agent | Software on a network device (switch, router, server) that responds to SNMP requests |
| OID (Object Identifier) | Unique identifier for a managed data point (like CPU usage or fan speed) |
| MIB (Management Information Base) | The database/schema of OIDs that the manager can query |
| Community String | Acts like a password – allows read/write access to SNMP data |

SNMP Traps –

SNMP traps are unsolicited messages sent from an SNMP-enabled device to a management station, which notifies it of significant events or conditions. Traps enable proactive monitoring and alerting, allowing admins to respond quickly to potential issues.

MIB (Management Information Database) –

MIB’s are a collection of OID’s in a hierarchical structure that define the properties of various network entities that can be managed using SNMP.

SNMP v2c -

SNMP v2c (Simple Network Management Protocol version 2 community-based) is an extension of the original SNMP protocol, offering enhancements like increased security with community strings and bulk retrieval capabilities. It is widely used due to its simplicity and effectiveness in network monitoring and management, but it lacks robust security features, relying on plain text community strings for authentication.

SNMP v3 –

SNMP v3 is the most secure version of the Simple Network Management Protocol, providing important security enhancements over its predecessors. It supports strong authentication and encryption, significantly improving the security of network management operations.

Community Strings –

Community strings in SNMP act as rudimentary passwords, allowing access to a device's management information. They are used in SNMP versions up to v2c. These strings must be carefully managed and secured, as they are transmitted in clear text, posing a potential security risk.

Authentication in SNMP v3 -

SNMP v3 enhances security through robust authentication mechanisms that verify the identity of the source and destination before allowing access to network data. It supports multiple authentication methods, including MD5 and SHA, to provide better security controls compared to its earlier versions.

Syslog Collector –

A syslog collector is a dedicated tool used for gathering log data generated by devices within a network. Syslog has certain severity levels that are denoted by a number and a name. The lower the number, the more severe.

| Level | Severity Name | Description | Use Case Example |
| --- | --- | --- | --- |
| 0 | Emergency | System is unusable | Kernel panic, power failure |
| 1 | Alert | Immediate action required | Database corruption, failed RAID |
| 2 | Critical | Critical conditions | Hardware failure, disk full |
| 3 | Error | Error conditions | Failed service, interface down |
| 4 | Warning | Warning conditions | High memory usage, nearing disk full |
| 5 | Notice | Normal but significant event | Configuration change, restart notice |
| 6 | Informational | Informational messages | Successful login, interface up |
| 7 | Debug | Debug-level messages | Detailed troubleshooting logs |

DNSSEC (Domain Name System Security Extensions) –

DNSSEC protects the integrity and authenticity of DNS records. It adds digital signatures to DNS records using public key cryptography. This is used for validating that DNS responses have not been tampered with.

DoH (DNS over HTTPS) –

DoH encrypts DNS queries inside HTTPS traffic. This prevents things like DNS snooping, MITM DNS attacks. It uses TLS within HTTP.

DoT (DNS over TLS) –

DoT encrypts DNS queries using TLS directly, using TCP port 853. This provides network wide privacy and is ideal for enterprises, VPN services, and secure networks.

Forward Lookup Zone –

Translates domain names to IP addresses.

Reverse Lookup Zone –

Translates IP addresses to domain names.

Authoritative vs. Non-Authoritative –

An Authoritative DNS server holds the actual records for the domain name/zone.

A Non-Authoritative DNS server relays or caches results from another source.

Primary vs. Secondary Zones –

A primary zone holds a read-write copy of a zone file and is the master source for DNS records.

A secondary zone holds a read only copy and is meant for backups/load balancing and fault tolerance.

Recursive Query –

A recursive query is when a DNS client asks a DNS resolver to find the complete answer to a domain name request and does not return a result until it has the final IP address or an error.

Hosts File –

The host’s file is a local text file on a computer that maps domain names to IP addresses manually before a DNS lookup is attempted.

Split Tunnel vs. Full Tunnel –

In a split tunnel VPN config, only network traffic for the corporate site passes through while other traffic accesses the internet directly. This reduces the load on the VPN gateway but increases security risks.

In a full tunnel VPN config, all the client’s internet traffic is routed through the VPN to the corporate network. This increases security as all traffic is encrypted but can lead to higher bandwidth usage.

4.0 Network Security

RADIUS (Remote Authentication Dial-In User Service) –

RADIUS is a centralized AAA (Authentication, Authorization, and Accounting) protocol often used for network access control. Widely used by ISP’s and enterprises to manage access to the network, keeping track of logging by users and ensuring their credentials are correct.

LDAP (Lightweight Directory Access Protocol) –

LDAP is an application protocol for accessing and maintaining distributed directory information services over an IP network. It is used to store info about users, groups, and devices, and supports strong authentication and encryption.

SAML (Security Assertion Markup Language) –

SAML is an open standard for exchanging authentication and authorization data between parties, specifically between an identity provider and a service provider. This standard allows identity providers to send proper authorization credentials to service providers, ensuring that user access is granted to appropriate resources based on pre-defined policies. Enables SSO.

TACACS+ (Terminal Access Controller Access-Control System Plus) –

TACACS+ is a protocol that handles authentication, authorization, and accounting services for networked access control. It separates these three functions which allow more flexibility in administration and provide better control over who can access what on the network.

5.0 Network Troubleshooting

Cat5 –

Up to 100 Mbps / 100m

Cat5e/Cat6 –

Up to 1 Gbps+ / 100m

Cat6a/Cat7 –

10 Gbps/ 100m

Crosstalk –

Signal bleeds into adjacent wires due to poor shielding or untwisted pairs. CRC errors or improperly crimped cable.

CRC errors (Cyclic Redundancy Check) –

Caused by electrical interference, damaged cables, and bad NIC’s.

Runts –

Packets that are smaller than 64 bytes.

Giants –

Packets larger than expected MTU (1500 bytes).

PoE (Power over Ethernet) –

Enables ethernet cables to carry both data and electrical power to devices.

Single Mode Fiber –

Used for long-distance transmissions, single mode fibers have a smaller core and support higher bandwidth with less signal attenuation.

Multimode Fiber –

Suitable for shorter distances, multimode fibers have a larger core, which allows multiple light modes but can cause more signal dispersion and attenuation over longer distances.

| Standard | IEEE Name | Max Power to Device (PD) | Voltage Range | Common Devices |
| --- | --- | --- | --- | --- |
| PoE | 802.3af | 15.4W (12.95W at PD) | 44–57V | IP phones, basic WAPs |
| PoE+ | 802.3at | 30W (25.5W at PD) | 50–57V | HD cameras, dual-band WAPs |
| PoE++ | 802.3bt Type 3 | 60W (51W at PD) | 50–57V | PTZ cameras, LED lighting |
| PoE++ | 802.3bt Type 4 | 100W (71–90W at PD) | 52–57V | Laptops, video systems |

FCoE (Fibre Channel over Ethernet) –

FCoE is a storage networking protocol that encapsulates Fibre Channel frames over ethernet networks, allowing storage and data traffic to coexist on a single unified network. FCoE operates at Layer 2 on the OSI model.

Additional Notes/Topics

Network Orchestration –

Orchestration in networking refers to the automation of network device configurations. It involves centrally managing and controlling networks devices, such as switches and routers, through SDN controllers or network management systems.

VIP (Virtual IP) –

VIP is shared among multiple devices, usually in a redundant configuration, to ensure continuous connectivity. VIP is not specific to a physical interface but rather shared among devices.

F-Type Connector –

Standard for coaxial cables, commonly used for cable modem connections.

Defense in Depth –

Defense in depth is a concept of layering various network appliances and configurations to create a more secure and defensible architecture.

802.1Q Tagging –

Crucial for VLAN management as it allows for traffic from multiple VLAN’s to be carried over the same physical link while keeping traffic segregated and properly managed.

Product Configuration –

Product Configuration refers to the tested and approved settings and parameters that a device should have when it is operating in a live, production environment.

Spectrum Analyzer –

A device that displays a single amplitude (strength) as it varies by signal frequency. This tool is used to identify connectivity issues with wireless connectivity.

North-South Traffic –

North traffic is exiting the datacenter, and South traffic is entering the datacenter.

East-West Traffic –

East West traffic refers to data flow within the data center.

PKI (Public Key Infrastructure) –

PKI manages digital certificates, including their issuance, revocation, and verification, providing a framework for secure communication and authentication.

Symmetric Key Encryption –

Symmetric Key Encryption uses a single key for both encryption and decryption, which doesn’t address the issues of authentication and certificate management.

NAT Gateway –

NAT Gateway allows resources within a VPC to access the internet without being directly exposed to the internet, ensuring a higher level of security for private instances.

VNI (VXLAN Network Identifier) –

A 24-bit identifier that helps differentiate between different VXLAN overlay networks. Each VXLAN segment or overlay network typically has a unique VNI assigned to it, allowing VXLAN endpoints to correctly identify and route traffic within the appropriate overlay network.

Teredo Tunneling –

Teredo is a transition technology that gives full IPv6 connectivity for IPv6-capable hosts that are on the IPv4 Internet but have no native connection to an IPv6 network. A private IP address is an IP address reserved for internal use behind a router or other Network Address Translation (NAT) devices, apart from the public.